IPC4BF2R4-DL2

4MP AI-ISP Fixed Bullet Network Camera

Key Features

- High quality imaging with 4MP resolution

- AI-ISP for excellent noise reduction effect

- 24/7 colorful imaging via AI-ISP technology

- High quality imaging with 4MP resolution, 1/1.8" CMOS sensor and F1.0 large aperture lens

- Efficient H.265+ compression technology

- Clear imaging against strong backlight due to 120 dB WDR technology

- Focus on human and vehicle targets classification

- Water and dust resistant (IP67)

- Built-in microphone for real-time audio security

Dimensions

Specification

| Camera |  |
| --- | --- |
| Image Sensor | 1/1.8" Progressive Scan CMOS Sensor |
| Min. Illumination | Color: 0.0005 Lux @ (F1.0, AGC ON) ,0 Lux with Warm light on |
| Illumination Distance | 30m (warm light) |
| Smart IR | N/A |
| Pan/Tilt/Rotation Range | Pan: 0° ~ 360° Tilt: 0° ~ 90° Rotation: 0° ~ 360° |
| Lens |  |
| Focal Length | 2.8/3.6 mm |
| Aperture | F1.0 |
| FOV | 2.8 mm @ F1.0 Horizontal FOV: 110°, Vertical FOV: 59°, Diagonal FOV: 130° 3.6 mm @ F1.0 Horizontal FOV: 95°, Vertical FOV: 51°, Diagonal FOV: 113° |
| Lens Mount | M16 |
| DORI |  |
| DORI | 2.8 mm @ F1.0, D: 57 m, O: 23 m, R: 11 m, I: 6m 3.6 mm @ F1.0, D: 68 m, O: 27m, R: 14m, I: 7m |
| Compression Standard |  |
| Video Compression | H.264/H.264+/H.265/H.265+/MJPEG |
| H.264 Codec Profile | Base/Main/High profile |
| H.265 Codec Profile | Main Profile |
| Video Bit Rate | 11 Kbps ~ 10Mbps |
| Triple Streams | Support |
| Audio Compression | G.711A, G.711U |
| Audio Bit Rate | 64 Kbps |
| Image |  |
| Max. Image Resolution | 4M (2688 × 1520) |
| Main Stream | 50Hz: 20fps 4M (2688 × 1520) 25fps 4M (2560 × 1440) 25fps 1080P (1920 × 1080) 60Hz: 20fps 4M (2688 × 1520) 30fps 4M (2560 × 1440) 30fps 1080P (1920 × 1080) |
| Sub Stream | 50Hz: 25fps 720P (1280 × 720) 25fps D1 (704 × 576) ,25fps CIF(352 × 288) 60Hz: 30fps 720P (1280 × 720) 30fps D1 (704 × 480) , 30fps CIF (352 × 240) |
| Third Stream | 50Hz:25fps CIF (352 × 288) 60Hz: 30fps CIF (352 × 240 ) |
| Image Setting | Brightness/Contrast/Saturation/Hue/Sharpness |
| Shutter Time | Auto/Manual, 1/3 ~ 1/100000s |
| Day/Night | IR Cut Filter |
| Wide Dynamic Range | 120dB |
| Digital Noise Reduction | 2D/3D DNR |
| BLC | Support |
| HLC | Support |
| ROI | Support |
| Rotate | Support |
| Smart Feature |  |
| Basic Event | Motion detection, video tampering |
| Smart Event | Smart motion detection, line crossing, intrusion, region entrance, region exiting (support alarm triggering by specified target types (human and vehicle) ) |
| Face Detection | N/A |
| Network |  |
| Network Storage | NAS (NFS, SMB/CIFS) |
| Protocols | TCP/IP, ICMP, HTTP, HTTPS, FTP, DHCP, DNS, DDNS, RTP, RTSP, RTCP, NTP, IGMP, UPnP, SMTP, UPnP-TM, Cloud |
| System Compatibility | Onvif (Profile S, Profile G, T) , SDK |
| Security | HTTPS encryption, IP address filter, basic and digest authentication for HTTP/HTTPS, TLS 1.1/1.2, WSSE and digest authentication for Open Network Video Interface |
| User | Up to 40 users |
| Web Browsers | Plug-in required live view: IE 10, IE 11 Local service: Chrome 57.0+, Firefox 52.0+ |
| Mobile Surveillance | IOS, Android |
| Active Deterrence |  |
| Red/Blue Light Warning | N/A |
| Sound Warning | N/A |
| Interface |  |
| Communication Interface | 1 RJ45 10 M/100 M self-adaptive Ethernet port |
| On-board Storage | Up to 256GB |
| MIC | Support |
| Speaker | N/A |
| Audio I/O | N/A |
| Alarm I/O | N/A |
| Reset Button | Support |
| General |  |
| Operating Conditions | (-40°C ~ 60°C) (-40°F ~ 140°F) Humidity 95% or less (non-condensing) |
| Power Supply | DC12V ± 25% |
| POE | PoE (802.3 af) |
| Power Consumption | MAX. 8W |
| Protection Level | IP67 |
| Material | Metal |
| Dimensions | Φ76 × 215.8 mm |
| Gross Weight | 900 g |

Accessory

Optional

| AC-JB02 | AC-PM03 | AC-CMB02 | AC-PC06 |
| --- | --- | --- | --- |

Installation Methods

| Junction Box | Pendant Mounting | Vertical Pole Mounting | Corner Mounting |
| --- | --- | --- | --- |
| IPC4BF2R4+JB02 | IPC4BF2R4+JB02+CMB02 | IPC4BF2R4+PM03 | IPC4BF2R4+JB02+PC06 |