IPC8MSD2R8-I2

2 x 4MP Dual-Directional PanoVu Camera

Key Features

- High quality imaging with 4 MP resolution

- 3D DNR technology delivers clean and sharp images

- Supports 2-channel WDR to realize clear imaging against strong back light

- Efficient H.265+ compression technology

- Water and dust resistant (IP67) and vandal resistant (IK10)

- Built-in dual-microphones for real-time audio security

- Easy installation with Power over Ethernet (PoE) technology

Dimensions

Specification

| Camera |  |
| --- | --- |
| Image Sensor | 1/2.7" Progressive Scan CMOS Sensor |
| Min. Illumination | Color: 0.006 Lux ( F1.6, AGC ON ) , 0 Lux with IR lights on |
| Illumination Distance | 30m ( IR light ) |
| Strobe Light | N/A |
| Smart IR | Support |
| Pan/Tilt/Rotation Range | Pan: -90° ~ 90° Tilt: 0° ~ 60° Rotation: 0° ~ 355° |
| Lens |  |
| Focal Length | 2.8mm |
| Aperture | F1.6 |
| FOV | 2.8mm@F1.6 Horizontal FOV: 101°, Vertical FOV: 55°, Diagonal FOV: 121° |
| Lens Mount | M12 |
| DORI |  |
| DORI | 2.8mm@F1.6, D:71m , O:28.4m , R:14.2m , I:7.1m |
| Compression Standard |  |
| Video Compression | H.264/H.264+/H.265/H.265+/MJPEG(sub stream） |
| H.264 Codec profile | Base/Main/High profile |
| H.265 Codec profile | Main Profile |
| Video Bit Rate | 83 Kbps ~ 10Mbps |
| Triple Streams | Support |
| Audio Compression | G.711A/G.711U |
| Audio Bit Rate | 64Kb |
| Image |  |
| Max. Image Resolution | 4MP(2688 × 1520) |
| Main Stream | 50Hz: 25fps 4MP(2688 × 1520) 25fps 4M(2560 × 1440) 25fps 1080P(1920 × 1080) 60Hz: 30fps 4MP(2688 × 1520) 30fps 4M(2560 × 1440) 30fps 1080P(1920 × 1080) |
| Sub Stream | 50Hz: 25fps 720P (1280 × 720) 25fps D1 (704 × 576) ,25fps CIF(352 × 288) 60Hz: 30fps 720P (1280 × 720) 30fps D1 (704 × 480) , 30fps CIF (352 × 240) |
| Sub Stream | 50 Hz: 25 fps (704 × 576, 640 × 480) 60 Hz: 30 fps (704 × 480, 640 × 480) |
| Third Stream | 50 Hz: 25 fps (1920 × 1080, 1280 × 720, 704 × 576, 640 × 480) 60 Hz: 30 fps (1920 × 1080, 1280 × 720, 704 × 480, 640 × 480) |
| Image Setting | Brightness/Contrast/Saturation/Hue/Sharpness |
| Shutter Time | Auto/Manual, 1/3~1/100000s |
| Day/Night | IR Cut Filter |
| Wide Dynamic Range | 120 dB |
| Digital noise reduction | 2D/3D DNR |
| BLC | Support |
| HLC | Support |
| ROI | Support |
| Rotate | Support |
| Smart Feature |  |
| Basic Event | Motion detection, Video Shelter |
| Smart Event | Motion detection (support alarm triggering by specified target types (human, vehicle, and bike) ) Line crossing, intrusion, region entrance, region exiting (support alarm triggering by specified target types (human, vehicle, and bike)) |
| Face Detection | Support |
| People Counting | Support one channel |
| Loitering Detection | Support |
| Network |  |
| Network Storage | NAS ( NFS, SMB/CIFS ) ,ANR |
| Protocols | TCP/IP, ICMP, HTTP, HTTPS, FTP, DHCP, DNS, DDNS, RTP, RTSP, RTCP, NTP, IGMP, UPnP, SMTP, UPnP-TM, Cloud |
| System compatibility | Onvif (Profile S，G，T), API |
| Security | Onvif (Profile S，G，T), API |
| Web Browsers | Plug-in required live view: IE 10, IE 11 Plug-in free live view : Chrome 57.0+, Firefox 52.0+/Safari Local service: Chrome 57.0+, Firefox 52.0+ |
| User | Up to 40 Users |
| Mobile Surveillance | IOS, Android |
| Active Deterrence |  |
| Red/Blue Light Warning | N/A |
| Sound Warning | Support |
| Interface |  |
| Communication interface | 1 RJ45 10M/100M self-adaptive Ethernet port |
| On-board storage | up to 256GB |
| MIC | Support |
| Speaker | Support |
| Audio I/O | 1/1 |
| Alarm I/O | 1/1 |
| Reset button | Support |
| General |  |
| Operating Conditions | ( -40°C ~ 60°C ) ( -40°F ~ 140°F ) Humidity 95% or less ( non-condensing ) |
| Power Supply | DC12V ± 25% |
| POE | PoE ( 802.3 at ) |
| Power Consumption | MAX. 12W |
| Protection Level | IP67,IK10 |
| Material | Metal |
| Dimensions | 202 × 105 × 71.2 mm |
| Gross Weight | 1130 g |

Accessory

Optional

| AC-JB16 | AC-CMB02 |
| --- | --- |

Installation Methods

| Junction Box | Pendant Mounting |
| --- | --- |
| IPC8MSD2R8+JB16 | IPC8MSD2R8+JB16+CMB02 |